| [YOUR CONSTRUCTION BUSINESS] |  |
| --- | --- |
| FINAL WALK-THROUGH CHECKLIST |  |
| The walk that writes the punch list — every room, the same checks, before the keys change hands |  |
| Project: ______________________________ Date: ____________ Walked by: ______________________ |  |
| Rooms walked: ______________________________ Items found (to the punch list): ________ |  |
| EVERY ROOM |  |
| ☐ | Walls and ceilings: nail pops, cracks, sanding marks, paint holidays, touch-ups that don't match |
| ☐ | Trim and doors: caulk lines, miters, doors latch and swing clear, stops in, hardware tight |
| ☐ | Windows: open, lock and seal; screens in; glass unscratched; sills caulked |
| ☐ | Floors: squeaks, gaps, scratches, transitions level, grout lines full, base shoe nailed |
| ☐ | Outlets, switches, lights and fans: every one works, plates level, bulbs in, dimmers smooth |
| ☐ | Smoke and CO detectors in place and tested; thermostats set and labeled |
| KITCHEN AND BATHROOMS |  |
| ☐ | Cabinets: doors aligned, drawers glide, pulls straight, shelves and kick plates in |
| ☐ | Countertops: seams tight, edges finished, caulk to the wall clean |
| ☐ | Faucets, drains and shutoffs: run every one, check under every sink for drips |
| ☐ | Tubs, showers and toilets: filled and drained, grout and caulk complete, no rocking |
| ☐ | Appliances: installed, leveled, running, manuals and tags collected for the owner |
| MECHANICAL, ELECTRICAL AND PLUMBING |  |
| ☐ | Furnace, air handler and water heater running; filters new; registers open and labeled |
| ☐ | Panel labeled; every breaker holds; GFCI and AFCI trip and reset |
| ☐ | Dryer vent, range hood and bath fans vent outside and the ducts are connected |
| EXTERIOR |  |
| ☐ | Siding, trim and flashing complete; paint and caulk finished; no exposed fasteners |
| ☐ | Roof: shingles flat, ridge vent on, gutters and downspouts hung and draining away from the house |
| ☐ | Grading slopes away; walks, steps and railings solid; porch lights and doorbell work |
| ☐ | Garage door opens, closes and reverses on the safety beam |
| ☐ | Site clean: debris gone, dumpster off, stakes pulled, landscaping as the plan shows |
| BEFORE THE KEYS CHANGE HANDS |  |
| ☐ | Every item found is on the punch list with a location, a trade and a number |
| ☐ | Keys, remotes, codes and manuals in one envelope |
| ☐ | Touch-up paint labeled by room; spare tile and flooring stacked for the owner |
| ☐ | Photos of every finished room, the same angles as the progress photos |
| Walk it with a flashlight and a roll of blue tape: tape every item as it's found, then write the list from the tape. One sheet per walk; the punch list is the sheet that comes out of it. |  |
| [Your construction business] · [Your phone] · [Your website] |  |

Free from zarla.com/guides/punch-list-template. Replace every [bracketed] line. Put your latest jobs on your website with the photos from the daily report, and send the quote and the invoice from the same place: zarla.com/website-builder/construction