| [YOUR BUSINESS NAME] |  |
| --- | --- |
| CAR DETAILING CHECKLIST |  |
| Express wash, interior, exterior and full detail — what's included, step by step |  |
| Customer: ____________________ Vehicle: ____________________ Date: ________ |  |
| Package (circle): express wash · interior · exterior · full detail |  |
| Add-ons: ________________________ Detailer: ______________ |  |
| BEFORE YOU START — EVERY PACKAGE |  |
| ☐ | Walk around the car with the customer and fill in the vehicle inspection form |
| ☐ | Photograph every existing scratch, chip, dent and stain |
| ☐ | Note personal items and ask the customer to take valuables |
| ☐ | Agree on the package, the add-ons and the price before you touch the car |
| ☐ | Check for warning lights and anything that doesn't work |
| EXTERIOR WASH — EXPRESS, EXTERIOR AND FULL |  |
| ☐ | Pre-rinse the whole car to lift loose dirt |
| ☐ | Wheels, tires and wheel wells first, with their own brushes and bucket |
| ☐ | Foam pre-wash, then a two-bucket hand wash from the roof down |
| ☐ | Door jambs, trunk jamb and fuel door |
| ☐ | Rinse, then dry with clean towels or a blower — mirrors and trim last |
| ☐ | Clean the outside glass, streak-free |
| ☐ | Remove tar, sap and bug residue |
| PAINT AND PROTECTION — EXTERIOR AND FULL |  |
| ☐ | Clay bar the paint to lift bonded contaminants |
| ☐ | Polish or paint correction, if booked |
| ☐ | Apply wax or sealant and buff off |
| ☐ | Dress exterior plastic trim — no shine on steps or pedals |
| ☐ | Dress the tires evenly, no sling |
| ☐ | Headlights checked, restored if booked |
| INTERIOR — INTERIOR AND FULL |  |
| ☐ | Remove floor mats and liners, clean them separately |
| ☐ | Vacuum seats, carpets, under the seats and the trunk |
| ☐ | Compressed air or a brush through vents, seams and cup holders |
| ☐ | Shampoo or extract carpets and mats |
| ☐ | Clean and condition leather, or shampoo fabric seats |
| ☐ | Dashboard, console and door panels wiped and protected, not greasy |
| ☐ | Steering wheel, shifter, handles and touch points cleaned |
| ☐ | Clean the inside glass, streak-free, including the top edge |
| ☐ | Headliner spot-cleaned only |
| ☐ | Pedals, seat tracks and door sills |
| ☐ | Odor treatment, if booked |
| ☐ | Mats and liners back in, seats and mirrors back where the customer had them |
| FINAL CHECK — EVERY PACKAGE |  |
| ☐ | Walk the car in daylight — streaks, missed spots, drips from mirrors and badges |
| ☐ | Check the glass again from inside and out |
| ☐ | No product on the windshield, pedals or steering wheel |
| ☐ | Radio, seat and settings left as the customer had them |
| ☐ | Take the after photos beside the before photos |
| ☐ | Keys and personal items handed back |
| ☐ | Explain the aftercare and when to book the next visit |
| Not included unless booked as an add-on: paint correction, ceramic coating, engine bay, headlight restoration, pet hair removal, odor treatment and stain removal. Price each as its own line on the quote. |  |
| [Your business name] · [Your phone] · [Your website] |  |

Free download from zarla.com/guides/car-detailing-checklist. Replace every [bracketed] line. Send the quote and the invoice from your own website: zarla.com/website-builder/car-detailing